# To do: experiment script update and vMMR analysis script

15 October 2026. Repository: https://github.com/DanielAtad/vMMR_EEG_PsychoPy_experiment. Line numbers refer to run_vMMR_experiment_v1_trig_after_framerate_measurement.py at commit 34fc744.

# Part 1 – Experiment script

Create run_vMMR_experiment_v2.py from run_vMMR_experiment_v1_trig_after_framerate_measurement.py (keep v1 unchanged, as the README asks). Items 1–5 are needed before data collection; 6–8 are recommended.

| # | Change | Why | Where (v1) |
| --- | --- | --- | --- |
| 1 | End the session with 255 (send 99 = end, then 255 = stop) | The recording model stops only on 255; with 99 it keeps recording until stopped by hand. | finish(final_code=99) :1364, set(99) :1367 |
| 2 | Preload images and read the trial tables before creating the LSL stream | Loading textures while the stream is live can pause PsychoPy; a pause > 0.1 s is a resync and shifts everything after it (the 0.8 s stall found in pilot testing). Same rule as for the window/refresh measurement. | stream created :1097; CSVs :1236–1238; preload_images :1254 |
| 3 | Write each face event's LSL timestamp to the events file (column lsl_event_timestamp) | Already captured (onset["lsl"], :552) but not saved. Lets the analysis match each events-file row to its corrected event exactly instead of relying on order. | make_event_writer fields :387–390; present_face_event return; event_writer.writerow :898 |
| 4 | Experiment-start marker 1, flip-aligned, when SPACE ends the instructions | Same anchor as the calibration scripts and the AGENTS.md map; marks the start after the model is running. Keep 9 (script start, not flip-aligned) or drop it. | trigger.set(9) :1283; instructions :1285 |
| 5 | Photodiode square off by default (photodiode_square); photodiode test trigger not 99 | The square must not appear for participants. PHOTODIODE_TEST_TRIGGER = 99 reuses the end code (and would stop an old model). | dialog :954; :110 |
| 6 | Write to run_info: git commit, file hashes, software versions, Windows refresh setting | Traceability for the methods section; the 120 Hz incident of 14 October was found only by chance. | run_info block :1150–1190 (as in run_photodiode_latency_database.py) |
| 7 | Optional: prime-onset (51–62) and fixation (80) markers via win.callOnFlip | Listed in AGENTS.md but not sent; useful for prime-locked analyses and epoch checks. Not needed for the vMMR. | run_trial :828 |
| 8 | Optional: show lsl_resyncs on the end screen | Immediate check by the experimenter instead of opening run_info. | end screen :1334 |

Validation after the update (before the first participant):

Dry run with the AGENTS.md checklist (dialog, practice and main trials, ESC, Hebrew, preloading, one events row per face, targets marked, square only on face-on frames when enabled).

1–3 full-length sessions with the photodiode square on face-on frames and the sensor at the bottom-right corner (the existing square position, so faces are not covered). Apply correct_events, compare with the optical onsets per face event: expected +2.0 ms (corner, from the 13 October sessions), SD ≈ 6 ms; also lsl_resyncs 0, model stopped by 255, all codes 11–44 present, events-file timestamps matched. Then set the square off for real sessions.

# Part 2 – vMMR analysis script

Nothing exists yet besides two notebooks that only load a June test file. Proposed structure (Python/MNE, or the lab's EEGLAB/FieldTrip pipeline with the same steps; decide first):

| Step | Content | Open decisions |
| --- | --- | --- |
| 1 Load | EEG .mat (EEG_data: col 0 time, EEG channels, optical sensor, marker) → MNE Raw with channel names, units and montage | Channel order/names and units of the g.HIamp configuration; cap layout |
| 2 Events | marker_timing.correct_events (+ QC); keep the main phase; match to *_events.csv (by LSL timestamp after Part 1 item 3, until then by code order); attach metadata (prime, identity, role, isAnalysisStandard, isAnalysisDeviant, target trials) | Session exclusion if QC fails |
| 3 Preprocessing | Filtering, 50 Hz line noise, bad channels, re-reference, ICA for eye artifacts | Filter cut-offs, reference, ICA settings – follow the original paradigm or preregistration |
| 4 Epochs | −100 to +500 ms around corrected face onset, baseline −100–0 ms (README); 3rd standard vs deviant, non-target trials only; artifact rejection | Rejection thresholds / autoreject |
| 5 ERPs | Per participant: 2 × 2 (prime DEATH/NEGATIVE × identity SELF/OTHER) standard and deviant ERPs; vMMR = deviant − standard | ROIs and time windows |
| 6 Statistics | Group level: repeated-measures analysis of window means and/or cluster-based permutation tests | Primary test as preregistered |
| 7 Behaviour | Target hit rate, false alarms, RT from *_trials.csv; participant exclusion criteria | Criteria |
| 8 Output | Per-participant QC report (n epochs per condition, rejected epochs, timing QC), group figures, tables; one config file; tests on a short real recording | BIDS export? |

Timing in the analysis: use only the corrected onsets; the per-event timing SD (5.7 ms, within one frame) is the same in all conditions and acts as slight smoothing. Prime-locked analyses should note that the prime appears about 4 ms before the centre (estimate).